## Using MIDAS for Icesheet Modeling

MIDAS provides a structured workflow for setting up and running 3D thermomechanical icesheet models for stratified ice deposits with different material types in non-terrestrial environments. MIDAS uses a forked version of ISSM, that can be found here: https://github.com/KBF-I/MIDAS/tree/Gen-1. Installing and setting up MIDAS is the same as ISSM. However, the subfolder called MIDAS will have to be added to MATLAB path. While ISSM offers interfaces to MATLAB and Python, MIDAS only offers MATLAB interface. Having set up ISSM, and to use MIDAS, one can follow these steps in MATLAB.

##### 1. Initializing a Model

To start a MIDAS simulation, initialize a new model using the modelmulti_Ice class:

model = modelmulti_Ice();

This automatically configures model settings for multi-layered icesheets.

##### 2. Defining Model Domain & Mesh

Defining the area domain and resolution will be the same as in ISSM. However, to properly initialize the modelling environment, use the following command. The currently acceptable bodies include EARTH, MARS, PLUTO, EUROPA, but the support can be extended to other planets by adding to the modelmulti_Ice library.

model = modelmulti_Ice.setPlanetMoonProperties('MARS');

At this point, we can continue with parameterization of the model, as it is done in ISSM.

##### 3. Extruding the 3D Icesheet Model

Once a 2D mesh is generated, MIDAS transforms it into a 3D stratified model. This is done differently from ISSM that uses the “extrude()” command:

model = extrudeModel(model, 1);

The second parameter indicates the multi-unit extrusion approach. Other value is “2” indicating the single unit extrusion. In ISSM, the number of vertical mesh layers used to build the 3D mesh is specified directly by the user. In MIDAS, however, the number of layers is determined by the thickness of the ice mass, within a range of values, since the icesheet may be composed of several distinct ice units. MIDAS defines the range such that the minimum number of layers per ice unit is always two (top and bottom mesh layers; see Figure 1 (2)), while the maximum of mesh layers in an ice unit is set by the user (or defaults to five if no value is provided). During extrusion, MIDAS selects the appropriate number of layers from this range, based on the unit’s average thickness. For every increment of average thickness (user-defined, with a default of 25 m), the number of layers increases from the base value of two up to the maximum allowed. As the model evolves, the number of layers assigned to a unit can therefore change over time. Once the layer count is determined, each ice unit is extruded separately, using ISSM’s logic to generate the full 3D multi-unit ice sheet.

##### 4. Configuring Surface Mass Balance (SMB) & Climate Parameters

SMB inputs in ISSM (and therefore in MIDAS) define accumulation rates per node over time. These inputs can be set via parametrized input files. The SMB values can be static or vary through time and space, depending on the year of the model and the location of a node. These can be positive (i.e., accumulation) or negative (i.e., ablation) and be from any of CO2, H2O or N2. MIDAS interpolates SMB values (in the x-y plane) if input data and surface coordinates do not align. It should be specified that it is also possible to change SMB according to its height if the SMB is assumed at a specific elevation (i.e. sublimation would be a different value at different elevations).

To apply SMB the addNewUnit function is used. This function will determine if the SMB of a new material type and a new unit needs to be created, or if SMB should be applied to an existing ice unit.

md = md.multiIceMesh.addNewUnit(md, tempUnit, periodSMB);

where periodSMB is an array of SMBs; and tempUnit is a new unit of ice and is created and can be initialized using:

tempUnit=stratified_unit();

tempUnit.Bed         = …              the elevation of the ice unit’s bed

tempUnit.Surface   = …              the elevation of the ice unit’s surface

tempUnit.IceType  = …              ice type such as CO2

tempUnit.ID           = …              an identification number assigned to the unit

Note that the bed and the surface are setup the same way that the bed and surface of the icesheet is setup in ISSM.

Surface temperatures can be defined for different time periods using preloaded climate data.

##### 5. Material Properties

MIDAS assigns material properties at the element level to enable transition across ice layers. Unlike ISSM, which assumes static ice properties, MIDAS supports temperature-dependent density, latent heat, thermal conductivity and heat capacity for icesheets.

The matmulti_ice.m script defines these properties for N₂, CO₂, and H₂O ice by default. Researchers can modify or extend this file to include additional materials like CH₄, CO, or H₂O with dust/rock; or replace the equations that are currently used in the file.

MIDAS, through the matlmulti_ice class, exposes various methods to retrieve different material properties. For example, to access the properties for an element, or a range of elements we can use:

properties = matmulti_ice.matPropLst4ElmntRange(md, propertyCONST,iceType, elementID_s)

where md is the model, propertyConst is the name of the property (e.g., “density”), iceType is the type of ice (e.g., “CO2”), and elementID_s is an element’s identification number, or a range of elements in form of [start_ID:end_ID]. The names of material properties are kept in mat_const.m and are values such as LatentHeat,  ThermalConductivity, HeatCapacity, Rho_Ice etc. MIDAS applies these properties automatically to elements during model setup, so there is no need for any action. Table 1 lists the modelling properties (including material properties) that need to be define by the user versus the properties that are controlled by MIDAS.

| Material Property | Property Type |
| --- | --- |
| Geo Flux | To be defined by the user |
| Heat Capacity | Calculated and controlled by MIDAS |
| Ice Density | Calculated and controlled by MIDAS |
| Latent Heat | Calculated and controlled by MIDAS |
| Melting Point | Controlled by MIDAS |
| Mesh Resolution | To be defined by the user |
| Thermal Conductivity | Calculated and controlled by MIDAS |
| TimeStep for Solving a Model | To be defined by the user |
| Rheology Law - Glen’s Exponent | Defined in MIDAS |
| Rheology Law - Aconst | Defined in MIDAS |
| Rheology Law - Activation Energy (Q) | Defined in MIDAS |

Table 1 – List of material properties used by MIDAS.

All properties controlled by MIDAS have been identified to distinguish with the properties that are user defined. In case any of the properties controlled by MIDAS needs changing, the user will have to manually modify the matmulti_Ice.m file and replace the hardcoded equation or value. Note that ISSM uses more material properties.

It should be mentioned that numerical stability requires that the chosen timestep satisfy Courant–Friedrichs–Lewy (CFL) constraints, meaning that information (e.g., advection of ice thickness or temperature) should not propagate across more than one mesh element per timestep. In practice, this limits the maximum timestep that can be applied, especially in regions with fine mesh resolution or high ice velocities.

##### 6. Running Transient Simulations

MIDAS uses the adaptive time stepping mechanism from ISSM for solver intervals. The minimum and maximum values are determined by the user, however, the maximum suggested time step for the solver interval is 5 years. Tests and comparisons between the results from MIDAS and ISSM for one type of material demonstrate that the use of time intervals larger than 5 years will result in the divergence of the results. However, time steps lower than 5 years does not provide increased accuracy; therefore the optimal value for computational efficiency as well as accuracy is 5 years. The solver dynamically determines the best step size, up to the maximum time stepping. Comparative tests, evaluating model thickness adjustments at intervals of 1, 2, 4, and 5 years and running transient models, have demonstrated that the results fall within an acceptable error range. Specifically, Table 5 presents variations in ice flow velocity within a 64,000-year model using a single material type, comparing changes in model thickness at the aforementioned intervals. This equates to the backend solver being invoked 64,000, 32,000, 16,000, or 12,800 times, respectively.

| Test Scenario | Maximum Velocity ([m.yr-1]) |
| --- | --- |
| Solving the model for every 1 year | 0.139549 |
| Solving the model for every 2 years | 0.139550 |
| Solving the model for every 5 years | 0.139551 |

Table 5-Test model, and the resulting maximum ice flow velocities if the surface mass balance is adjusted every 1, 2 or 5 years.

The discrepancy in maximum ice flow velocity, observed by alternating Surface Mass Balances (SMBs) annually versus executing the solver on an annual or quinquennial basis, is minimal, quantified at 0.000009 [m.yr-1]. Extending the interval beyond five years precipitates a pronounced escalation in discrepancies, leading to a rapid divergence among models. It is pertinent to note that the Ice Sheet System Model (ISSM) incorporates a specialized logic for the application of SMBs, thereby enhancing computational efficiency and expedited processing. In contrast, MIDAS, which directly modifies the ice units' thickness, is unable to exploit the efficiencies inherent to ISSM, resulting in a decelerated pace of solving transient models. This deceleration is markedly evident in simulations extending over several hundred thousand years. For instance, a CO2ice unit accumulating at a rate of 2 [mm.yr-1]. would, upon solver updates every five years, exhibit an increment of 10 mm in thickness at each node.

Like ISSM, MIDAS runs the simulation as follows:

model = solve(model, model_type);

Where model_type carries the same parameters as those defined in ISSM (e.g., Thermal and Transient which indicate thermal or transient models respectively). MIDAS automatically handles transient failures, ensuring model continuity.

##### 7. Capturing & Analyzing Results

After solving the model, use captureResult to extract key outputs:

results = modelmulti_Ice.captureResult(model);

This function provides:

•	Ice thickness evolution

•	Flow velocity fields

•	Surface/basal temperature changes

To plot the results, all the plotting functionality from ISSM can be used. Moreover, MIDAS exposes a new plotting function to plot a 3D meshed view of the icesheet with the ice units identified (Figure 1):

plotting.plotmultiUnit_mesh(model); % Visualize stratified icesheet

MIDAS also offers a predefined set of scripts which receive an excel file as input and leverages MIDAS functionality to define and analyze icesheets.